# 5G Core Network-Based Video Streaming using FFmpeg and IP Camera

## Introduction

┌─────────────────┐

│  Video Source   │ Axis IP Camera (192.168.1.100)

│  RTSP Server    │ H.264/AAC encoding, RTSP/RTP

└────────┬────────┘

│ RTSP over TCP (Port 554)

│ Video: H.264, Audio: AAC

│

┌────────▼────────┐

│  Core Network   │ Media Processing Node (192.168.1.11)

│   (EPC/5GC)     │ Transcoding, QoS, Distribution

└────────┬────────┘

│

├─────────────────────┬──────────────────────┐

│                     │                      │

│ UDP/MPEG-TS         │ UDP/MPEG-TS         │

│ Port 1234           │ Port 5000           │

│ 3 Mbps              │ 3 Mbps              │

│                     │                      │

┌────────▼────────┐    ┌───────▼─────────┐

│   UE/Sender     │    │  RAN/Receiver   │

│ (192.168.1.12)  │    │ (192.168.1.10)  │

│  Endpoint 1     │    │  Endpoint 2     │

└─────────────────┘    └─────────────────┘

## 2. System Architecture

The high-level architecture consists of the following components:
• IP Camera (RTSP Source)
• Sender (FFmpeg encoder)
• 5G Core Network (AMF, SMF, UPF)
• UE Emulator or gNB
• Receiver (FFplay or FFmpeg client)

Setup required:
Core network:

netstat -an | grep 1234

netstat -an | grep 5000

UE:

netstat -an | grep 1234

RAN:

netstat -an | grep 5000

IP Camera setup:

firefox 192.168.1.100 &

## 3. IP Camera Streaming (RTSP)

The IP camera exposes its RTSP media stream at:
rtsp://Admin:Admin123@192.168.1.100/axis-media/media.amp

This URL is verified using:
  ffplay rtsp://Admin:Admin123@192.168.1.100/axis-media/media.amp

## 4. Sender: Transmitting Stream via UDP

The sender encodes RTSP stream and forwards it over UDP for transport across the core network:
ffplay -fflags nobuffer -flags low_delay udp://@:1234

## 5. Receiver: Playing UDP Stream

On the receiver side, the incoming stream is played with:
   ffplay -fflags nobuffer -flags low_delay udp://@:5000

## 6. Core Network

ffmpeg -rtsp_transport tcp -i rtsp://Admin:Admin123@192.168.1.100/axis-media/media.amp \
  -c:v libx264 -preset veryfast -tune zerolatency -b:v 3M -c:a aac \
  -f mpegts udp://192.168.1.12:1234 \
  -c:v libx264 -preset veryfast -tune zerolatency -b:v 3M -c:a aac \
  -f mpegts udp://192.168.1.10:5000

## 7. Common Issues and Fixes

• Address already in use → Caused by multiple ffplay instances binding to the same port.
  To resolve:
      pkill -9 ffplay

• 401 Unauthorized → Wrong IP camera username/password.

• No video → Ensure UPF rules allow UDP forwarding.